Day-3 Assignment

Q1. List 5 common issues users face with USB keyboards or mice on laptops, and write one possible solution for each.

Ans.

laptops, and write one possible solution for each.

5 Common USB Keyboard and Mouse Issues with Solutions

| No. | Common Issue | Possible Solution |
| --- | --- | --- |
| 1 | Device not detected: The keyboard or mouse is not recognized by the laptop. | Unplug the device and reconnect it to a different USB port. |
| 2 | Keyboard keys not responding: Some keys do not type anything. | Check for stuck keys and reconnect the keyboard. Test it on another computer if necessary. |
| 3 | Mouse cursor not moving: The mouse is connected but the cursor does not move. | Clean the mouse sensor and try using it on a suitable, flat surface. |
| 4 | Intermittent connection: The device repeatedly disconnects and reconnects. | Check the USB cable for damage and try another USB port. |
| 5 | USB driver problem: The keyboard or mouse does not work correctly because of a driver issue. | Open Device Manager, locate the device, and update its driver or uninstall the device and restart the laptop. |

Q2. Simulate a troubleshooting flow: Your Bluetooth headphones are not connecting to your phone. Write the exact step-by-step actions you would take to diagnose and fix the issue, mentioning any settings or indicators you would check.

Ans.

Troubleshooting Flow: Bluetooth Headphones Not Connecting to a Phone

Problem: Bluetooth headphones are not connecting to a mobile phone.

Step-by-step troubleshooting procedure

Step 1: Check the headphones' battery

- Turn on the headphones.

- Check the battery indicator or charging LED.

- If the battery is low, charge the headphones and try again.

Step 2: Enable Bluetooth on the phone

- Open Settings → Bluetooth on the phone.

- Turn Bluetooth ON.

- Check that Bluetooth is enabled and the phone can detect nearby devices.

Step 3: Put the headphones into pairing mode

- Turn off the headphones.

- Press and hold the power or Bluetooth button according to the manufacturer's instructions.

- Check whether the LED flashes, often blue or alternating blue and red, indicating pairing mode.

Step 4: Search for the headphones

- Open the phone's Bluetooth settings.

- Select Pair new device or Available devices, if shown.

- Wait for the headphone name to appear.

- Tap the correct device name to connect.

Step 5: Check the connection status

- Confirm that the headphones show Connected in the Bluetooth settings.

- Play a short audio clip and check whether sound comes through the headphones.

- If the phone asks for a pairing confirmation or PIN, follow the manufacturer's instructions.

Step 6: Disconnect other devices

- Check whether the headphones are already connected to another phone, laptop, or tablet.

- Disconnect them from that device, then repeat the pairing process.

Step 7: Forget and pair again

- Open Settings → Bluetooth.

- Tap the settings icon next to the headphone name.

- Select Forget, Unpair, or Remove device.

- Put the headphones back into pairing mode and pair them again.

Step 8: Restart both devices

- Turn Bluetooth off and on again on the phone.

- Restart the phone.

- Turn the headphones off and on, then try connecting again.

Step 9: Reset the headphones

- If the problem continues, perform the manufacturer's recommended factory-reset procedure.

- Put the headphones into pairing mode and try connecting again.

Step 10: Test the headphones with another device

- Try pairing the headphones with another phone or computer.

- If they connect successfully, the original phone may have a Bluetooth settings or software issue.

- If they fail to connect to multiple devices, consult the headphone manufacturer's support instructions.

Troubleshooting summary

| Check | Expected Indicator |
| --- | --- |
| Battery | Sufficient charge or normal charging light |
| Phone Bluetooth | Bluetooth is switched ON |
| Pairing mode | Pairing LED flashes or voice prompt is heard |
| Device list | Headphone name appears under available devices |
| Connection | Status displays Connected |
| Audio test | Sound plays through the headphones |

Q3. A user reports that their external webcam is not detected during a Zoom call on Windows 10. Write a checklist of 6 things to check or try to resolve the problem.( Hint: Consider device manager, privacy settings, and app permissions.)

Ans.

Here is a 6-step troubleshooting checklist to resolve an external webcam not being detected in Zoom on Windows 10:

1. Check Physical Cables and Ports

- Disconnect the webcam's USB cable and reinsert it firmly into a different USB port on your PC (preferably a direct port on the motherboard/chassis rather than an unpowered USB hub).

- If your webcam features a physical privacy shutter or built-in toggle switch, make sure it is completely open.

2. Verify Windows 10 Privacy Settings & App Permissions

- Go to Settings > Privacy > Camera.

- Ensure Allow access to the camera on this device is turned On.

- Toggle Allow apps to access your camera to On, and scroll down to ensure Allow desktop apps to access your camera (which includes Zoom) is enabled.

3. Check Device Manager for Hardware Recognition

- Right-click the Start menu and open Device Manager.

- Expand the Cameras, Imaging devices, or Sound, video and game controllers sections.

- Check if the webcam is listed. If a down arrow appears on the icon, right-click it and select Enable device.

4. Update or Reinstall Webcam Drivers

- In Device Manager, right-click your webcam and select Update driver > Search automatically for drivers.

- If updating does not work, select Uninstall device, unplug the webcam, restart your PC, and plug the webcam back in so Windows can automatically reinstall fresh drivers.

5. Confirm Selected Camera & Permissions in Zoom

- Open Zoom, click your profile icon at the top right, and go to Settings > Video.

- Under the Camera dropdown menu, ensure your external webcam is selected instead of a virtual camera or integrated laptop camera.

- Verify that no other application (like Microsoft Teams, Skype, or Google Meet) is open and locking the webcam stream in the background.

6. Adjust Antivirus & Web Protection Settings

- Open your antivirus or third-party security software (e.g., Kaspersky, Bitdefender, Norton).

- Locate the Webcam Protection or Privacy Control settings to ensure Zoom is listed as a trusted application and is not actively blocked from accessing your camera hardware.

Q4. You are helping a friend whose printer is showing 'offline' status on their PC. Describe how you would use the Windows Troubleshooter tool to try to fix the issue, and explain what information the tool provides during the process.

Ans.

Troubleshooting a Printer Showing “Offline” in Windows

Objective: Use the Windows Troubleshooter to identify and resolve the problem preventing the printer from communicating with the PC.

Step-by-step procedure

1. Check the printer: Make sure the printer is switched on, has paper, and displays no error messages. Check the USB cable or Wi-Fi connection.

2. Open Windows Settings: Press Windows + I to open Settings.

3. Open Troubleshooting settings: In Windows 10, navigate to Update & Security → Troubleshoot → Additional troubleshooters.

4. Run the Printer Troubleshooter: Select Printer, click Run the troubleshooter, and follow the on-screen instructions.

5. Review the results: Allow Windows to check for common printer problems, such as connection issues, incorrect configuration, or problems with the print spooler service.

6. Apply the recommended fixes: If the troubleshooter identifies a problem, follow its instructions to apply the suggested solution.

7. Test the printer: Open Settings → Devices → Printers & scanners, select the printer, and check its status. Print a test page to confirm that it is working.

What information does the Troubleshooter provide?

| Information | Explanation |
| --- | --- |
| Problems detected | Identifies possible causes of the printer being offline. |
| Connection status | Helps identify potential communication problems between the PC and printer. |
| Print spooler issues | May detect problems with the Windows service that manages print jobs. |
| Configuration issues | May identify incorrect settings or a printer that is not configured properly. |
| Fixes applied | Reports whether Windows automatically resolved any detected problems. |
| Further recommendations | Provides suggested actions when the problem cannot be fixed automatically. |

Expected result: The printer reconnects to the PC and changes from Offline to Ready or Online.

Note: The Troubleshooter may not detect or fix every problem. If the printer remains offline, check whether Use Printer Offline is enabled in the print queue, clear stuck print jobs if necessary, and verify the printer driver and connection.